Development and Characterization of Tl- and Ce- doped novel Organic Glass Scintillator for Radiation Detection

J.Y. Cho1, D.W. Jeong1, J.H. Kim1, H. J. Kim2, *

1The Center for High Energy Physics, Kyungpook National University, 41566, Daegu, Republic of Korea

2Department of Physics, Kyungpook National University, 41566, Daegu, Republic of Korea

Corresponding Author Email: hongjooknu@gmail.com

Scintillators are important for X-ray imaging, homeland security, astrophysics, and high-energy and particle physics. In practice, it is still nontrivial to obtain large-area scintillators that simultaneously satisfy high optical transparency, shape flexibility, and low-cost fabrication.

Ethyltriphenylphosphonium bromide (ETPBr) based organic/inorganic hybrid glasses can be produced by a conventional melt–quenching process at relatively low temperature, providing shape flexibility while maintaining high optical transparency. Motivated by previous reports on the high light yield of (ETP)₂MnBr₄ glasses [1], we note that their long decay time limits their applicability to pulse-counting radiation detectors. In this work, we aim to develop a Tl- and Ce-doped novel ETPBr-based organic glass scintillator that combines high light yield with fast scintillation decay.

Tl- and Ce- doped novel ETPBr based organic glass scintillators were fabricated using melt–quenching, followed by optimization of composition and heat-treatment conditions to obtain transparent samples. Their scintillation performance and optical properties were characterized by X-ray–induced luminescence, light yield, scintillation decay time, photoluminescence quantum yield (PLQY), and optical transmittance. These results indicate that this novel organic glass scintillator can be applied to medical X-ray imaging and other scintillator-based applications.

[1] Li, Bohan & Xu, Yan & Zhang, Xinlei & Han, Kai & Jin, Jiance & Zhiguo, Xia. (2022). Zero‐Dimensional Luminescent Metal Halide Hybrids Enabling Bulk Transparent Medium as Large‐Area X‐Ray Scintillators. Advanced Optical Materials