Implementation of a Novel Pure LaCl3 Scintillation Detector for Hard X-Ray Diagnostics in Thailand Tokamak-1

Sawarin Buakham1,*, Kamontip Ploykrachang2, Siriyaporn Sangaroon1,3, Kunihiro Ogawa3,4, Kitti Rongpuit5, Nopporn Poolyarat6, Arlee Tamman6, Hong Joo Kim7, Nguyen Duy Quang8,

Jakrapong Kaewkhao9, Nuanthip Wantana9, Mitsutaka Isobe1,3,4

1Faculty of Science, Mahasarakham University, Maha Sarakham, Thailand

2Faculty of Engineering, Chulalongkorn University, Bangkok, Thailand

3National Institute for Fusion Science, National Institutes of Natural Sciences, Toki, Japan

4The Graduate University for Advanced Studies, SOKENDAI, Toki, Japan

5King Mongkut’s University of Technology Thonburi, Bangkok, Thailand

6Thailand Institute of Nuclear Technology (Public Organization), Bangkok, Thailand

7Department of Physics, Kyungpook National University, Daegu, South Korea

8Dalat Nuclear Research Institute, Dalat, Viet Nam

9Center of Excellence in Glass Technology and Material Science (GCEM), Nakhon Pathom Rajabhat University, Nakhon Pathom, Thailand

Corresponding Author Email: 67010252002@msu.ac.th

Figure 1. Measured X-ray energy spectra obtained with the pure LaCl3 scintillation detector [2].

Enhancing the understanding of runaway electron behavior in fusion plasmas is important because runaway electrons can cause significant damage to plasma-facing components and pose a critical challenge to the safe operation of fusion devices. Following the first plasma discharge in July 2023, Thailand Tokamak-1 (TT-1)—the first small-sized tokamak in Thailand—has advanced toward achieving high-performance plasma operation and is being developed as a national platform for fusion plasma physics experiments. During ohmic plasma discharges, hard X-ray emissions were observed using LaBr3(Ce) scintillation detector operated in pulse mode [1] and plastic scintillation detector operated in current mode. The hard X-ray signals appear during the plasma start-up phase, persist throughout the flat-top phase, and then rapidly decrease at plasma collapse.

Although LaBr3(Ce) have demonstrated good performance for hard X-ray spectroscopy, further development of novel scintillation detectors is required to meet the demanding operational conditions of fusion plasma experiments. Recently, a newly developed pure LaCl3 scintillation detector has been investigated for its feasibility in hard X-ray measurements (see Figure 1) [2]. In this work, the implementation of a pure LaCl3 scintillation detector system for hard X-ray spectroscopy on the TT-1 is presented. The first hard X-ray spectral measurements obtained using a pure LaCl₃ scintillation detector will be reported in this work. These results demonstrate the feasibility of pure LaCl3 scintillators for hard X-ray diagnostics in fusion plasmas and provide a foundation for the further development and application of advanced scintillation-based diagnostic systems.

- K. Rongpuit, et al., “Initial results of hard X-ray spectroscopy by LaBr3(Ce) detector for runaway electron study in Thailand Tokamak-1”, Radiation Physics and Chemistry 227, 112346, 2025.

- K. Ploykrachang, et al., “Feasibility and characterization of a pure LaCl3 scintillation detector for hard X-ray measurements in Thailand Tokamak-1”, Radiation Physics and Chemistry 240, 113471, 2026.

This research project was financially supported by the Program Management Unit for Human Resources and Institutional Development, Research and Innovation, PMU-B fiscal year 2025 (Grant No. B39G680008).