Optimization of Nano-scale TiO2 Reflectors via Atomic Layer Deposition for High-Resolution Scintillation Detectors

Sangsu Kim1, Chansun Park1, Hyojung Kim2, Mingu Kim5, Shinhaeng Cho2, Jung-Yeol Yeom3,4,*

1Global Health Technology and Research Center, Korea University, Seoul, Republic of Korea

2Department of Radiation Oncology, Chonnam National University Medical School, Gwangju, Republic of Korea

3Department of Bioengineering, Korea University, Seoul, Republic of Korea

4School of Bioengineering, Korea University, Seoul, Republic of Korea

5Department of applied physics, Korea University, Sejong, Republic of Korea

Corresponding Author Email: jungyeol@korea.ac.kr

Fig. 1. Comparison between the simulated reflectance curve of ALD TiO2 and discrete experimental data points for Teflon tape, ESR film, and TiO2 paint. The ALD thin film achieves high reflectance at only 250 nm thickness, offering a 1,600-fold reduction in thickness compared to the 400 μm Teflon reference.

High-resolution scintillation detector arrays require ultra-thin reflectors to minimize dead space and maximize the fill factor. In this study, we investigate the feasibility of sub-micron titanium dioxide (TiO2) thin films, deposited via Atomic Layer Deposition (ALD), as a replacement for conventional bulky reflector materials. Based on our previous experimental results [1], widely used reflectors such as Teflon tape, ESR film, and TiO2 paint require substantial thicknesses—up to 400 μm, 65 μm, and 138 μm, respectively—to achieve optimal reflectance. Optical simulations performed using the Transfer Matrix Method (TMM) indicate that an ALD-deposited TiO2 film with a thickness of only 250 nm can achieve a reflectance of 95.7%, which is comparable to that of a 400 μm-thick Teflon reflector. This enhanced performance is attributed to the high refractive index of the dense TiO2 phase (n ~2.5) and the elimination of the interfacial air gaps. As shown in Fig. 1, the ALD-based nano-reflector reaches its reflectance saturation at a thickness approximately three orders of magnitude thinner than that of conventional reflector materials. These findings demonstrate that ALD-fabricated nano-reflectors can significantly reduce detector volume while preserving high light collection efficiency, thereby enabling the development of next-generation, high-resolution scintillation detectors for medical imaging systems. Further experimental validation and detailed analyses will be presented at the conference.

References

- C. Park et al., “Compatibility of TiO2 reflective material with Ce:GAGG scintillators in harsh environments,” Optical Materials, 157, 116165 (2024).

Acknowledgments

This work was supported by the National Research Foundation of Korea (RS-2023-00240040, RS-2023-00234651, RS-2023-NR077190, RS-2025-00561749), Chonnam National University’s Hwasun Hospital Institute for Biomedical Science (HCRI23-034), and Korea University Grant.