Fast Fabrication of Eu2O3-Doped Silicate Glass Scintillator Prepared by Microwave Melting Technique

N. Singkiburin1, N. Srisittipokakun1,2,*, N. Kiwsakunkran1,2, N. Intahcai3, S. Kothan3, H.J. Kim4,

J. Kaewkhao1,2

1Physics Program, Faculty of Science and Technology, Nakhon Pathom Rajabhat University, 73000, Thailand

2Center of Excellence in Glass Technology and Materials Science (CEGM), Nakhon Pathom Rajabhat University, Nakhon Pathom 73000, Thailand

3Center of Radiation Research and Medical Imaging, Department of Radiologic Technology, Faculty of Associated Medical Sciences, Chiang Mai University, Chiang Mai, 50200, Thailand

4Department of Physics, Kyungpook National University, Daegu, 41566, South Korea

Corresponding Author Email: nattapon@webmail.npru.ac.th

Photoluminescence emission spectra of Eu2O3-doped silicate glasses. (ex=394 nm)

A series of Eu2O3-doped SiO2:Na2O:BaO:ZnO glasses was fast prepared using a microwave melting technique to investigate their structural, photoluminescence (PL), and radioluminescence (RL) properties for scintillation application. The amorphous nature of the samples was confirmed by X-ray diffraction (XRD). UV–Vis–NIR spectra provided information on optical transparency and Eu3+-related absorption features. Under optical excitation (λex = 394 nm), the glasses exhibited characteristic Eu3+ emission, and the PL intensity increase with increasing Eu2O3 concentration, indicating enhanced activation of luminescent centers. The emission chromaticity lay in the red region with CIE (x, y) = (0.6458, 0.3538). Changes in the local symmetry around Eu3+ ions were evaluated using the electric-dipole to magnetic-dipole (ED/MD) intensity ratio and further supported by Judd-Ofelt (JO) analysis. Under X-ray excitation, the samples showed scintillation light, with maximum intensity at 3.00 mol% and integral radioluminescence efficiency is 8.9% relative to a commercial BGO scintillator. These results demonstrating promising potential for scintillation and x-ray screen.