Design and Development of a ZnS:Ag-based Fast-Ion Loss Probe for the Compact Helical Device

Sheng Dong1, Kunihiro Ogawa1,2, Longyong Liao2, Worathat Paenthong1, Yinghan Wu1, Ruijie Zhou3, and Mitsutaka Isobe1,2,4

1The Graduate University for Advanced Studies, SOKENDAI, Toki, Gifu, Japan

2National Institute for Fusion Science, National Institutes of Natural Sciences, Toki, Gifu, Japan

3 Institute of Plasma Physics, Chinese Academy of Sciences, Hefei, China

4 Mahasarakham University, Maha Sarakham, Thailand

Corresponding Author Email: dong.sheng@nifs.ac.jp

With the steady advancement of magnetic confinement fusion devices, including tokamaks and stellarators, the realization of deuterium–tritium fusion power is becoming increasingly attainable. To sustain stable and efficient burning plasmas, a comprehensive understanding and accurate diagnosis of energetic particles (EPs) behavior are essential for optimizing plasma performance.

The Compact Helical System (CHS), a stellarator originally operated by the National Institute for Fusion Science, is now being upgraded following the planned shutdown of the Large Helical Device (LHD) at the end of 2025. The upgraded device, referred to as the Compact Helical Device (CHD), is scheduled to begin operation in late 2026.

Scintillator-based diagnostics have been extensively applied on CHS and LHD for direct measurements of escaping energetic ions. In particular, ZnS:Ag scintillators are widely used due to their high light yield for impact of energetic ions, robustness under harsh plasma environments, and cost-effectiveness. For instance, a ZnS:Ag scintillator-based lost-fast-ion probe (SLIP) was first implemented on CHS in 1997 to detect escaping beam ions’ energy and pitch angle [1-2], and was subsequently utilized as a magnetic spectrometer to investigate energetic-ion transport due to magnetohydrodynamic instabilities in CHS [3] as well as in LHD [4-5]. In addition, ZnS:Ag scintillators have been successfully applied in imaging neutral particle analyzer (INPA) to diagnose confined EPs [6] and EP transport due to energetic-particle-driven resistive interchange.

Motivated by these experiences, the development of ZnS:Ag-based fast-ion loss diagnostics is being continued for CHD. This work aims to improve the understanding of EPs confinement in CHD plasmas characterized by three-dimensional magnetic field structure, and the design and development progress of the SLIP for CHD will be presented.

1.	M. Isobe et al., “Escaping fast ion diagnostics in compact helical system heliotron/torsatron”, Review of Scientific Instruments 70, 827–830 (1999).

2.	M. Isobe et al., “Energy and pitch angle-resolved measurements of escaping helically trapped energetic ions at the small major radius side of the compact helical system”, Review of Scientific Instruments 74, 1739–1742 (2003).

3.	M. Isobe et al., “Studies of fast-ion transport induced by energetic particle modes using fast-particle diagnostics with high time resolution in CHS”, Nuclear Fusion 46, S918 (2006).

4.	K. Ogawa et al., “Observation of energetic-ion losses induced by various MHD instabilities in the Large Helical Device (LHD)”, Nuclear Fusion 50, 084005 (2010).

5.	K. Ogawa et al., “Magnetic configuration effects on TAE-induced losses and a comparison with the orbit-following model in the Large Helical Device”, Nuclear Fusion 52, 094013 (2012).

6.	W. Paenthong et al., “Design and initial results of the imaging neutral particle analyzer in large helical device", Review of Scientific Instruments 95, 083547 (2024).