Quenching-Factor Determination for (n,p) and (n,α) Reactions in CLYC Using KRISS Mono-energetic Neutrons

Sinchul Kang1, Jungho Kim2, Joong Hyun Kim2, Hyeonseo Park2, Hyeoungwoo Park3,

Young Soo Yoon2

1Institute for Rare Isotope Science, Institute for Basic Science, Daejeon, Republic of Korea

2Korea Research Institute of Standards and Science, Daejeon, Republic of Korea

3Central Research Institute, Korea Hydro & Nuclear Power Co. Ltd., Daejeon, Republic of Korea

Corresponding Author Email: sckang@ibs.re.kr

Cs2LiYCl6:Ce (CLYC) scintillators enable neutron-gamma discrimination through pulse-shape discrimination (PSD) [1-2]. In addition, for fast neutrons in the few MeV ranges, neutron-induced charged-particle reactions in CLYC provide signals whose amplitudes scale with neutron energy, making the detector suitable for neutron spectrometry.

We measured mono-energetic neutrons generated at the Korea Research Institute of Standards and Science (KRISS). The facility uses an electrostatic accelerator to accelerate deuterons and produces mono-energetic neutrons via the D(d,n) reaction on a TiD target [3]. Deuteron energies of 150 keV and 200 keV were used, and neutrons of various energies were measured by changing the measurement angle from 0° to 149° with respect to the beam direction. The CLYC detector was calibrated using gamma-ray sources to establish the electron-equivalent energy scale. Mono-energetic neutrons over multiple energies were measured using the CLYC detector. Based on the measured results and the gamma-ray calibration, quenching factors for the (n,p) and (n,α) reaction channels in CLYC were derived.

In this presentation, we will report the measurement of mono-energetic neutrons at various energies using CLYC and discuss the detector characteristics inferred from these results.

1.	C.M. Combes et al., “Optical and scintillation properties of pure and Ce3+-doped Cs2LiYCl6 and Li3YCl6:Ce3+ crystals”, J. Lumin, B 82, 299 (1999)

2.	J. Glodo et al., “Scintillation Properties of 1 Inch Cs2LiYCl6:Ce Crystals”, IEEE Trans. Nucl. Sci., 55, 1206 (2008)

3.	Sinchul Kang et al., “Neutron irradiation facilities, neutron measurement system, and mono-energetic neutron fields at KRISS”, J. Korean Phys. Soc., 82, 586 (2023)